# Interfacing 4x4 Matrix Keypads with STM32

A 4x4 keypad is the bread and butter of user input for microcontrollers. Instead of hogging 16 separate pins for 16 buttons, it uses a clever "matrix" trick to get the job done with only 8 pins.

# 1. How a 4x4 Keypad Works

The keypad consists of 16 momentary switches arranged in 4 rows and 4 columns.

- The Matrix Trick: The switches are not connected to a common ground. Instead, each button connects one Row line to one Column line when pressed.

- The Scanning Process:

  - The microcontroller sets all Rows as outputs and all Columns as inputs (with internal pull-up resistors).

  - It drives one Row LOW at a time.

  - While a Row is LOW, it checks the state of the Columns.

  - If a Column is detected as LOW, the microcontroller knows exactly which button is pressed because it knows which Row it just "activated."

# 2. Pin Diagram & Description

A standard membrane or tactile 4x4 keypad typically has 8 pins sticking out of a single ribbon cable.

The 8 pins on the ribbon cable are divided into two groups: Rows and Columns. This setup allows the microcontroller to "scan" which button is being pressed by checking the intersection of a row and a column.

Here is the breakdown of the pins

| Pin Number | Label | Function | Connected Buttons (Keys) |
| --- | --- | --- | --- |
| 1 | R1 | Row 1 Control | 1, 2, 3, A |
| 2 | R2 | Row 2 Control | 4, 5, 6, B |
| 3 | R3 | Row 3 Control | 7, 8, 9, C |
| 4 | R4 | Row 4 Control | *, 0, #, D |
| 5 | C1 | Column 1 Control | 1, 4, 7, * |
| 6 | C2 | Column 2 Control | 2, 5, 8, 0 |
| 7 | C3 | Column 3 Control | 3, 6, 9, # |
| 8 | C4 | Column 4 Control | A, B, C, D |

### How to Read the Diagram

Think of these pins as the "coordinates" for each button. Because it's a matrix, no single button has its own dedicated wire. Instead:

- Rows (Pins 1-4): These are typically configured as Outputs on your microcontroller.

- Columns (Pins 5-8): These are typically configured as Inputs with internal pull-up resistors.

Example of how it works in practice:

If you press button "5", it creates a physical bridge between Pin 2 (R2) and Pin 6 (C2). Your code detects that while it was looking at the second row, the second column suddenly changed state.

# 3. Connecting to STM32

While you can use almost any General Purpose I/O (GPIO) pins, it is easiest to keep them in a single port (like Port A or Port B) to simplify coding.

Example Connection (STM32F4 or L4 Series):

- Rows (Outputs): PA0, PA1, PA2, PA3

- Columns (Inputs): PA4, PA5, PA6, PA7

Note: In your STM32CubeIDE configuration, set the Column pins (PA4-PA7) to Input mode with Pull-up resistors enabled. Set the Row pins (PA0-PA3) as Output Push-Pull.

# 4. Sample C Code (Basic Scanning)

This is a simplified logic snippet for an STM32 using the HAL library.

char keymap[4][4] = {

{'1','2','3','A'},

{'4','5','6','B'},

{'7','8','9','C'},

{'*','0','#','D'}

};

char Keypad_Scan(void) {

for (int i = 0; i < 4; i++) {

// Set all rows HIGH

HAL_GPIO_WritePin(GPIOA, GPIO_PIN_0|GPIO_PIN_1|GPIO_PIN_2|GPIO_PIN_3, GPIO_PIN_SET);

// Pull current row LOW

HAL_GPIO_WritePin(GPIOA, (uint16_t)(1 << i), GPIO_PIN_RESET);

if (HAL_GPIO_ReadPin(GPIOA, GPIO_PIN_4) == GPIO_PIN_RESET) return keymap[i][0];

if (HAL_GPIO_ReadPin(GPIOA, GPIO_PIN_5) == GPIO_PIN_RESET) return keymap[i][1];

if (HAL_GPIO_ReadPin(GPIOA, GPIO_PIN_6) == GPIO_PIN_RESET) return keymap[i][2];

if (HAL_GPIO_ReadPin(GPIOA, GPIO_PIN_7) == GPIO_PIN_RESET) return keymap[i][3];

}

return 0; // No key pressed

}

# 5. References

- Link: https://controllerstech.com/use-4x4-keypad-with-stm32/

- Link: https://www.youtube.com/watch?v=xs-2kqyzmPU