Project Report 1
SG90 Servo Motor Interfacing with STM32

Course: Microprocessor & Embedded System,Section:08

Name: Mobussira Tasnim Pushpita

ID:2311032042

Note: SG90 is an actuator/peripheral device converts a PWM control signal into controlled angular motion.

# 1. Description and Working Principle

The SG90 is a small 9 g micro servo motor widely used in robotics, automation, camera positioning, and embedded control systems. Unlike a normal DC motor, it does not rotate continuously. Instead, it moves to a commanded angular position and can hold that position.

Internally, the SG90 contains a DC motor, reduction gears, a control circuit, and a feedback potentiometer. The potentiometer reports the shaft position to the internal controller. The controller compares the desired position with the actual position and drives the motor until both match.

The servo is controlled by PWM. A control pulse is repeated about every 20 ms, which means 50 Hz. In common operation, a pulse near 1.0 ms moves the shaft close to 0°, 1.5 ms moves it near 90°, and 2.0 ms moves it near 180°. The exact end positions may vary slightly from one servo to another.

# 2. Pin Diagram and Pin Description

The SG90 has three wires for power and control.

| Wire Color | Pin Name | Description |
| --- | --- | --- |
| Brown / Black | GND | Connects to common ground |
| Red | VCC (+5V) | Supplies power to the servo motor |
| Orange / Yellow | Signal (PWM) | Receives the PWM control pulse from STM32 |

# 3. Connection with STM32

A timer PWM output pin of the STM32 is used to control the servo. A common lab setup is STM32F103C8T6 (Blue Pill) using PA0, which works as TIM2_CH1. The microcontroller generates the PWM signal and the SG90 interprets the pulse width to set its angle.

The servo should be powered from a stable 5 V source. The servo ground and STM32 ground must be connected together so that the signal has a proper reference.

| SG90 Pin | STM32 Pin | Purpose |
| --- | --- | --- |
| GND | GND | Common reference ground |
| VCC | 5V | Servo power supply |
| Signal | PA0 (TIM2_CH1) | PWM control output |

Working idea: configure the timer for 50 Hz PWM and change the compare value (CCR) to vary pulse width and therefore servo angle.

# 4. Sample STM32 C Code (HAL)

#include "main.h" extern TIM_HandleTypeDef htim2; void Servo_SetAngle(uint8_t angle) { uint16_t pulse = 25 + ((100 * angle) / 180); __HAL_TIM_SET_COMPARE(&htim2, TIM_CHANNEL_1, pulse); } int main(void) { HAL_Init(); SystemClock_Config(); MX_GPIO_Init(); MX_TIM2_Init(); HAL_TIM_PWM_Start(&htim2, TIM_CHANNEL_1); while (1) { Servo_SetAngle(0); HAL_Delay(1000); Servo_SetAngle(90); HAL_Delay(1000); Servo_SetAngle(180); HAL_Delay(1000); } }

# 5. Applications, Advantages and Limitation

Applications: robotic arm, camera pan-tilt system, smart door system, and small automation projects.

Advantages: simple 3-wire interface, accurate angle control, lightweight structure, and easy PWM-based operation.

Limitation: standard SG90 has a limited rotation range and should not be used for heavy mechanical loads.

# 6. Conclusion

The SG90 servo motor is a low-cost and practical actuator for position control in embedded systems. By using an STM32 timer in PWM mode, the motor can be controlled accurately and repeatedly. Its simple interfacing and useful motion control make it an excellent choice for academic projects and beginner robotics systems.

# 7. References

ControllersTech, “How to Control a Servo Motor with STM32 using PWM”- https://www.bing.com/ck/a?!&&p=d7f099f32c582ef7325da1d966c201ee3ac07a7ac2647119aa0fbe4706a33b79JmltdHM9MTc3NTAwMTYwMA&ptn=3&ver=2&hsh=4&fclid=0de28dc5-df1f-691f-07c4-9f02de1e68cc&psq=Controlling+a+Servo+Motor+with+STM32+Using+PWM+(Step-by-Step+Guide)&u=a1aHR0cHM6Ly9jb250cm9sbGVyc3RlY2guY29tL3NlcnZvLW1vdG9yLXdpdGgtc3RtMzIv

YouTube video: https://youtu.be/WMS0t9WGqVw?si=ZXtakJOJucYzfCGY